The University of Azad Jammu and Kashmir,

Muzaffarabad

Lab Title:                Basic Router Access and CLI Configuration

Course Name:         Database Systems

Lab No:                    06

Course Code:          CS-2203

Submitted to:          Engr. Dr. Asma Javaid

Student Name:       Kamal Ali Akmal

Roll No:                  2024-SE-38

Session:                   2024-2028

Semester:                4th

Submission date:    7th May 2026

Basic Router Access and CLI Configuration

Introduction

Network devices such as routers and switches must be configured before they can operate in a network. Configuration is usually done through the Command Line Interface (CLI). There are two main ways to interact with networking devices:

• GUI (Graphical User Interface)

• CLI (Command Line Interface)

In networking, CLI is widely used because it provides more control and flexibility.

Connecting to Network Devices

To configure routers or switches directly, a computer must be connected to the device using a console connection.

Common connection components:

• Console Port on router/switch

• RJ45 Console Cable

• DB9 Connector (older PCs)

• USB Mini Console Port (newer devices)

• Rollover Cable used for console connection

Task

Step 1: Open Router

- Go to Cisco Packet Tracer

- Drag a router

- Click it → go to CLI tab

Step 2: Enter Modes, Set passwords and Encrypt passwords

Router> enable
Router# configure terminal
Router(config)#

Router(config)# enable password 123
Router(config)# enable secret 456

If both exist → enable secret overrides password

Router(config)# service password-encryption

This converts visible passwords into encrypted form

Step 3: Check configurations

Router# show running-config
Router# show startup-config